Chapter Outline, Push System vs. Pull System

(Note:  This Chapter Outline corresponds to Chapter 10 within Operations Management: Customer-Focused Principles, Schonberger & Knod, 6th edition, 1997.  Numbers at right are the corresponding page numbers for your use in referencing.)

Executive Summary:  Modern Japanese operations use customer demand-driven "pull" systems such as kanban that communicate the need to process when consumption pulls material "up the line" from the preceding workstations (eg, empty McDonald's hamburger or french fry slots that 'drive' more production).  Traditional American operations use producer forecast-driven "push" systems such as MRP that communicate the need to process when material is pushed "down the line" into workstations towards customer demand.

FIX PAGE NUMBERS

Push Systems vs. Pull Systems						(316)

Push System

Provider pushes products "down the line" to customer demand

MRP is a push system						(318)

Pull Systems

Customer demand pulls products "up the line" from provider

Kanban is a pull system

Push Systems are, and have been, the dominant system in the US	(316)

Inflexibility/resistance of existing system

Distance from suppliers (vs. Japan's Toyoda City)

Underestimation of inventory carry costs

Queue

Means "waiting line"							(308)

Kanbans

Also called "queue limitation"						(308)

Items pulled through the system by user according to demand		(308)

Systematized at Toyota during 1950's by Ohno Taiichi, Dir. Mfg. Ops.	(-----)

Types of Kanbans

Cards									(-----)

a card is passed from user to provider

Containers								(310,311)

...which have cards attached

Squares								(310)

an empty area on floor/table that holds one unit of material

....which signals the need for production

Time Policy								(310)

maximum time that units/people allowed in queue

Units Policy								(311)

maximum number of units/people allowed in queue

Kanban Removal

Simple method for continuous improvement				(314)

Start with a higher number of kanbans with low quantities			(315)

Remove kanbans that are not needed; ie, fill rate is 100%		(315)

Benefits of Kanbans

Brings customer and supplier closer together				(308, 317)

Quicker throughput times						(312)

(313-314)

Better delivery/completion promises					(312)

(313-314)

MRP

Stands for "Material Requirements Planning"				(318)

Uses cummulative lead times to determine order releases		(319)

Computer programs suited to MRP "cascading" calculations		(320)

Calculations								(322-328)

COVERED BY TQ

Rescheduling Notice							(325)

Means "change in a end item due date or quantity"

Nervous Schedule							(326)

Means "lots of little changes caused by rescheduling notices"

Total response to very nervous schedules is impractical

The "available options" on MRP

TPOP									(326)

Stands for "Time Phased Order Point"

Used to include independant demand in MRP system

Example:  Replacement Windshields

DRP									(329)

Stands for "Distribution Requirements Planning"

Similar to MRP, but external					(-----)

Aggregates POS demand forecasts to drive MPS		(329)

µ §

Closed Loop MRP

Means "self-adjusting MRP for common changes"		(329)

Causes								(329)

Production floor goes off -schedule

End item due date changes

Adjustments							(330)

Reschedule supplier orders

Reschedule component parts on production floor

MRP II

Stands for "Manufacturing Resource Planning"			(330)

Extends MRP to other functional areas such as finance		(330)

Benefits of  MRP								(331)

Increased on-time completion

Lower Inventories

Allows workcenter capacity planning to be done

Cash flow planning

Reorder Point

means "get more when you only have X items left"			(332)

Calculation

COVERED BY TQ

Two Bin System								(332)

Variation of Reorder Point

Means "get some more when the first bin is empty"

Also bins with dividers, or bins with "bathtub ring," min/max

"The basic difference between pull and push is that a pull system initiates production as a reaction to present demand, while push initiates production in anticipation of future demand."  Karmakar, U. 1989.  Getting control of just-in-time.  Harvard Business Review.  67(5): 123