Chapter Outline, Continuous Flow and Repetitive Manufacturing

(Note:  This Chapter Outline corresponds to Chapter 13 within Operations Management: Customer-Focused Principles, Schonberger & Knod, 6th edition, 1997.  Numbers at right are the corresponding page numbers for your use in referencing.)

Executive Summary:  Products that are produced in these type of environments are more like price-sensitive commodities.  Success in these environments, therefore, tends to revolve around cost reduction (eg, investing in specialized equipment, minimizing material costs, locating plants close to vendors and customers) and overall consistency (eg, quality control, regular deliveries and schedules, preventing equipment failures).

FIX PAGE NUMBERS

Process Industries

Streamlined flow							(405, 389)

Frequently has repetitive mfg process at packaging stage		(404-5)

Example:  Potato Chips

Major Success Factors (in the past)

Capital Investment							(406)

Automate flows for high, continuous yields

Optimal mix (or the "Diet Problem" or the "Blending Problem")		(406)

Linear Programming (LP) models used for calculation

[*This is an example, you do not have to be able to DO an LP]

Suppose you want to make a ton of cat food ...

|  | Protein | Fat | $ / lb | lbs. Avail ? |
| --- | --- | --- | --- | --- |

| Beef (B) | 15% | 50% | $0.50 | 1,000 |
| --- | --- | --- | --- | --- |

| Chicken (K) | 10% | 32% | $0.40 | 1,000 |
| --- | --- | --- | --- | --- |

| Grain (G) | 1% | 2% | $0.13 | 1,000 |
| --- | --- | --- | --- | --- |

| Cardboard (C) | 0% | 0% | $0.01 | 200 |
| --- | --- | --- | --- | --- |

The "least-cost" solution without any constraints

is obvious -- Let 'em eat pure cardboard !

But there are constraints, either by marketplace

or gov't regulations.  Let's say they are:

minimum protein content of 11%

maximum fat content of 28%

amounts of material available for purchase

Now the situation is something like this:

Pure beef has too much fat (and not very cheap)

Pure chicken has too little protein

A 1000-1000 beef-chicken mix is too fat (41%) so some

maybe we should mix in some grain a/o cardboard

Try to use cardboard if possible to lower costs, but

there's not much available !

Not to worry, we have a mathematical problem to solve:

Minimize

(.50 B) + (.40 K) + (.13 G) + (.01 C)		'total costs

Subject to

B + K + G+ C = 2000				'total weight

(.15 B) + (.10 K) + (.01 G) + (.00 C) ³		'protein

.11(B + K + G + C)

(.50 B) + (.32 K) + (.02 G) + (.00 C) £		'fat

.28(B + K + G + C)

B £ 1000 ; K £ 1000; G £300			'mat. avail.

B ³ 0 ; K ³ 0 ; G ³ 0 ; C ³ 0			'nonnegative sol.

For continuous flow, saving even a few cents per ton

using the real "least-cost" mix, can save big bucks ITLR.

Strict maintenance							(407)

Breakdowns can idle whole plant

Breakdowns can spoil perishiable materials

Close process monitoring						(407)

Ensure consistancy / quality of product

Logistics								(407)

Effective location of plants, warehouses

Reliability of raw material delivery

Regularized Schedules							(407,411)

Slotting of major, high volume products

Improves predictibility for materials, production, & sales

Reduces related planning tasks

Major Success Factors (current / future)

Optimal mix								(408)

Use "what-If" LPs to suggest raw material improvements

Strict PREVENTIVE maintenance					(408)

Close process monitoring						(408)

SPC, automated, instrumented quality

Logistics								(408)

Strong partnerships with few suppliers

Speed of industry requires such mutualism		(-----)

Another topic that, though not directly related, fits better in this chapter than any other........

Line Balancing

Means "trying to distribute work equally across the workstations		(414)

Two types of line balancing are often required

Engineered line balancing					(414,417)

Sequenced (or scheduled) line balancing			(417)

µ §

A truck manufacturer wants its new plant to build 10 trucks per hour, or one truck every six minutes.  When designing the production floor, tasks on the engine assembly production line were engineering balanced (ie, arranged by the engineers) so that every workstation, on average, has the same amount of work to do per truck.

| Type of | Sales |  | Type of | Cycle Time (minutes) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Truck | Ratio |  | Truck | Carb | Wiring | Total |
| Type A | 50% |  | Type A | 8 | 4 | 12 |
| Type B | 50% |  | Type B | 4 | 8 | 12 |
| Total | 100% |  | Average | 6 | 6 |  |

After the plant is built, the company finds it is selling two types of trucks in equal amounts.  On Type A, the carburetor installation takes more time but wiring installation takes less time; the reverse is true for Type B.  Cycle time at the carburetor and wiring workstations average out to 6 minutes each in the long run.  It appears that have worked out nicely.

The Monday morning schedule:

A, A, A, A, A, A, B, B, A, A, A, .......

however, cause problems.  The production line is running at 6 minutes per engine, but the Type A carburetors are taking 8 minutes each.  Work begins to back up at the carburetor workstation.  The wiring workstation is idle 75% of the time; it works for 2 minutes, then waits 6 minutes for the next engine to arrive.  The line scheduler could have better balanced the line by writing the schedule:

A, B, A, B, A, B, A, B, A, B, ..........

Watching the Lights							(418)

A tactic for ad-hoc line balancing

(This is also a Japanese tactic called andon, which means lantern)

Americans consume 600 million pounds of potato chips annually; so it seems they are manufactured AND consumed as a continuous flow process !